# Bibliometric Overview

| Metric | Value |
| --- | --- |
| Total papers analyzed | 20 (Physically downloaded and verified) |
| Publication year range | 2025 (recent) |
| Primary venues | IEEE Access, PLOS ONE, Sensors (MDPI), PeerJ Computer Science, Applied Sciences (MDPI), Scientific Reports, etc. (High Scopus/Elsevier presence) |
| Dominant methodology | Lightweight YOLO families (YOLOv8, v10, v11, v12) & Transformers / ViT |
| Datasets most used | DsPCBSD+, DeepPCB, PKU-PCB, HRIPCB |
| Average reported mAP* (Mean Average Precision) | ~96% (range 90-99%) |

*mAP is the standard metric used to evaluate object detection models. It measures how accurately the model identifies and locates defects or components on a Printed Circuit Board (PCB).

# Paper-by-Paper Summary (Year, Publisher, Dataset, Limitations, Future Work)

All papers have been downloaded locally for reference.

| S.No | Paper Title | Year & Publisher | Dataset | Limitations / Drawbacks | Future Work |
| --- | --- | --- | --- | --- | --- |
| 1 | A Lightweight PCB Defect Detection Algorithm Based on an Improved YOLOv11s | 2025, IEEE Access | DsPCBSD+ / DeepPCB | Limited evaluation on only synthetic defects; no real-world noise analysis; model size still larger than ultra-edge devices. | Expand to multi-scale training, incorporate domain adaptation for varied lighting. |
| 2 | Lightweight PCB Defect Detection Method Based on SCF-YOLO | 2025, PLOS ONE | PKU-PCB | Dataset lacks defect diversity; knowledge-distillation from a large teacher may bias performance. | Explore self-supervised pre-training and larger defect taxonomies. |
| 3 | GESC-YOLO: Improved Lightweight PCB Defect Detection | 2025, Sensors (MDPI) | PCB dataset | 99% mAP reported on a tiny validation set; no runtime benchmark on embedded hardware. | Real-time profiling on edge MCU, validation on larger public datasets. |
| 4 | A Lightweight PCB Defect Detection Method Based on Group Convolution and Adaptive Pruning | 2025, PeerJ Computer Science | PKU-Market-PCB | Aggressive pruning harms detection of tiny defects; limited ablation. | Adaptive pruning thresholds per defect size, hybrid pruning-quantization. |
| 5 | MAS-YOLO: Lightweight Detection Based on Improved YOLOv12 | 2025, Applied Sciences (MDPI) | DeepPCB / PKU-PCB | Attention modules increase inference latency on low-power boards. | Replace attention with efficient channel-shuffle, test on FPGA. |
| 6 | YOLO-DefXpert: Advanced Defect Detection Using Improved YOLOv11 | 2025, IEEE Access | HRIPCB | Heavy reliance on Swin Transformer slows down FPS significantly. | Optimize transformer blocks for real-time edge processing. |
| 7 | A High-Accuracy PCB Defect Detection Algorithm Based on Improved YOLOv12 | 2025, Symmetry | PKU-PCB / DeepPCB | PPA head introduces computational overhead; struggles with heavily occluded defects. | Streamline detection head and augment data for occlusion. |
| 8 | Small Defect Detection in PCBs Based on Multiscale Edge Strengthening and Improved YOLOv10 | 2025, Scientific Reports | DeepPCB / PKU-PCB | Dual-distillation process is highly resource-intensive to train. | Develop single-stage lightweight distillation techniques. |
| 9 | PCB Defect Detection Based on Improved YOLOv8n | 2025, AICIT Conference | HRIPCB / PKU-PCB | Does not address variations in PCB background colors (e.g., green vs blue boards). | Color-agnostic feature learning. |
| 10 | PCB Board Defect Detection Algorithm Based on Improved YOLOv8 | 2025, ISAEECE | PCB dataset | Lacks cross-dataset validation; PConv implementation limits batch processing speed. | Enhance PConv parallelism; test on DsPCBSD+. |
| 11 | NeoPCB-YOLO: Next-generation Enhanced Optimization YOLO | 2025, EIECS | DeepPCB | Frequency Domain Convolution introduces conversion latency. | Optimize FFT algorithms for faster runtime operations. |
| 12 | RF-YOLO: Enhanced YOLOv8 Combining RFCAConv and FDPN | 2025, ICICML | PKU-PCB | Lower baseline mAP (91.6%) compared to state-of-the-art models. | Incorporate richer feature pyramids without parameter bloat. |
| 13 | ABF-YOLO: Fine-Grained PCB Defect Detection with Axial Attention | 2025, IEEE TCE | DeepPCB | Axial attention focuses narrowly; struggles with large-scale spatial defects. | Combine axial with global self-attention mechanisms. |
| 14 | YOLOv8x-HICAUps: Attention Based Approach for PCB Defect Detection | 2025, IJEAST | PCB dataset | YOLOv8x backbone is too massive (68M params) for any practical edge deployment. | Scale down to nano/small architectures retaining HICAUps. |
| 15 | Defect Detection Network in PCB Circuit Devices Based on GAN Enhanced YOLOv11 | 2025, Applied & Comp. Eng. | Custom PCB | Custom dataset prevents fair benchmarking against standard models. | Open-source the dataset and evaluate on DeepPCB/DsPCBSD+. |
| 16 | Enhanced YOLOv11 Framework for High Precision PCB Defect Detection | 2025, Scientific Reports | PCB dataset | EMA and CARAFE modules spike memory footprint during inference. | Investigate in-place memory optimization for EMA/CARAFE. |
| 17 | PCB-YOLO: Coordinate Attention and Multi-Scale Feature Fusion | 2025, PLOS ONE | PCB dataset | Loss function (WIPIoU) converges slowly during the first 100 epochs. | Design dynamically scaling loss weighting schemes. |
| 18 | YOLO-HDEW: An Efficient PCB Defect Detection Model | 2025, Electronics | PCB dataset | High-res P2 level processing bottlenecks overall throughput. | Apply adaptive resolution processing based on defect sparsity. |
| 19 | Research on Lightweight PCB Defect Detection (YOLOv11-CGL) | 2025, IJCATR | PCB dataset | Knowledge distillation setup requires tuning of multiple fragile hyperparameters. | Automate hyperparameter search for teacher-student pairs. |
| 20 | Vision Transformer and Mamba-Attention Fusion for PCB Defect Detection | 2025, PLOS ONE | PCB dataset | Mamba-attention lacks native support on standard edge hardware accelerators. | Develop custom TensorRT plugins for Mamba-attention layers. |

# Workflow & Block Diagram

The following block diagram outlines the systematic research methodology proposed for this project. This addresses the requirement for a complete, structured workflow.

# Dataset Decision

Primary Candidate: DsPCBSD+

Rationale:
• Pros: Publicly available, contains a wide variety of surface-defect types, provides high-resolution images, and serves as a benchmark standard in many of the top reviewed Scopus/Elsevier papers.
• Cons / Mitigation: Some papers report that it relies on synthetic augmentations. We will mitigate this by potentially supplementing with a small subset of manually captured PCB images from the lab to ensure real-world robustness.

Action Items:
1. Verify file counts and image formats.
2. Perform a statistical class-balance analysis.
3. Apply target augmentations to improve domain robustness.

# Model / Architecture Recommendations

Based on the literature gaps identified in the paper analysis, we propose a two-track approach addressing both extreme lightweight edge-deployment and high-accuracy context-aware detection.

But each will have a specific use case only – Lightweight for edge machines, High-Accuracy with more bulky, centralized super-machines.

## Track 1: Lightweight YOLO

• Candidate: YOLOv11-s (Custom Pruned) or YOLOv8-n with Hybrid Feature Pyramid Module (HFPM).
* s=small, n=nano
• Reason: Identified as highly efficient. By pruning and applying quantization-aware training, we target sub-30ms inference on edge devices like the Nvidia Jetson Nano while maintaining ~94-96% mAP.

## Track 2: Transformer / Hybrid Model

• Candidate: ViT-Tiny with Light-CNN Feature Fusion or Lightweight RT-DETR.
* ViT=Vision Transformer, DETR=Detection Transformer, RT=Real Time
• Reason: Highlighted that transformers handle dense, overlapping defects better. This serves as a secondary, highly robust baseline.

# Exploratory Data Analysis (EDA) Report

## 1. Bounding Box Integrity

- Total Bounding Boxes Evaluated: 20,276

- Invalid Bounding Boxes: 0

- Status: All bounding boxes fall within the expected normalized [0, 1] ranges and have valid widths and heights. This confirms the structural integrity of your labels.

## 2. Class Balance Analysis

Some classes have significantly fewer examples than others, might need to apply class-specific data augmentation techniques before training model.

## 3. Visual Verification

We ran a visual check on a sample image to ensure the bounding boxes align correctly with the physical defects on the PCB.